CIA Inspection Checklist

This list serves as a guideline for Colorado Inspection Agency inspectors to assure that important code issues are not overlooked and to provide uniformity in the inspection process.  It is not meant to encompass everything that will be looked at during the course of an inspection.

## FOUNDATION INSPECTIONS

Setbacks/General:

Pass|Fail

| FND.1 |  |  | Check site plan for required setbacks – from string line between property pins. |
| --- | --- | --- | --- |
| FND.2 |  |  | Check top of foundation elevation 12” plus 2% above street flow line or drainage inlet on property. |
| FND.3 |  |  | Review approved plan for footing size, reinforcing schedule and design notes. |

Footings:

Pass|Fail

| FND.4 |  |  | Check depth and width. |
| --- | --- | --- | --- |
| FND.5 |  |  | Check for minimum depth below frost line (30 inches) |
| FND.6 |  |  | The top surface of footings shall be level. |
| FND.7 |  |  | Forms should be substantially tight to prevent leakage. |
| FND.8 |  |  | If reinforcing is specified check for proper clearance from soil (3” above). |
| FND.9 |  |  | Check for proper compaction – no loose material, frost or mud. |

Foundation Walls/Piers:

Pass|Fail

| FND.10 |  |  | Check width and depth of walls or piers as indicated on the approved plans. |
| --- | --- | --- | --- |
| FND.11 |  |  | Check reinforcement is clean and free of mud or other debris. |
| FND.12 |  |  | Check reinforcing for proper clearance from forms, ground, etc. |
| FND.13 |  |  | Check bar splices, spacing, sizes and schedule. |
| FND.14 |  |  | Check reinforcing at windows and block-outs (continuous around corners for 24”). |
| FND.15 |  |  | Check void form to assure that no gaps between void forms exist that will allow soil to contact concrete. |
| FND.16 |  |  | Check footing to wall dowels or keyway. |
| FND.17 |  |  | Check for egress window in basements of dwellings. |

## UNDERGROUND PLUMBING

Pass|Fail

| UP.1 |  |  | Review plumbing plan for piping details. |
| --- | --- | --- | --- |
| UP.2 |  |  | Check to see if a backwater valve is required. |
| UP.3 |  |  | Check venting of floor drains and assure that floor drains are connected to sewer. |
| UP.4 |  |  | If vent for floor drain is a dry vent it should connect above the centerline of the horizontal drain pipe. |
| UP.5 |  |  | Check slope of drains to be ¼ inch per foot for 2 ½” diameter and less, if necessary 1/8 inch per foot for 3” to 6” and 1/16 inch per foot for 8” or larger. |
| UP.6 |  |  | Check for water or air leaks depending on test method (10’ head of water or 5 P.S.I. air). |
| UP.7 |  |  | Check for proper bedding of pipe (uniformly bedded not propped up on dirt clods, etc). |
| UP.8 |  |  | Check for proper horizontal fittings (sweep not vertical fittings). Horizontal to horizontal, quarter bends allowed for 2 inch or smaller pipe only. |
| UP.9 |  |  | Check for cleanouts at each change of direction greater than 45 degrees, except not more than one cleanout shall be required in each 40 feet regardless of change in direction. |
| UP.10 |  |  | Check to see if floor drain is through the backwater valve. If not, note on back of permit card to check floor drain for Flood guard at finals. |

### ROUGH FRAMING INSPECTION

Structural Frame:

Pass|Fail

| RF.1 |  |  | Building to be dried in (roofing felt at minimum fastened with battens or nails with washers). |
| --- | --- | --- | --- |
| RF.2 |  |  | Lateral bracing – if plywood or other panels are used for bracing method check nailing patterns and make sure all 4 edges are nailed properly from outside (block horizontal joints to accomplish this if necessary). Thermoply panels require roofing nails (see nailing pattern printed on panel). Panels are to be applied directly to the studs not over gyp sheathing or insulation. |
| RF.3 |  |  | Check for solid blocking framing corners in fire-rated construction. |
| RF.4 |  |  | Lateral bracing – steel angle braces are to extend from top plate to bottom plate and nailed per listing. The angle of the brace is to be no less than 45 deg. Nor more than 60 deg. from horizontal. |
| RF.5 |  |  | Check all bearing point loads such as at roof beams, girder trusses floor beams and make sure all bearing points are adequately supported through all levels to the foundation. Check to see that bearing posts on plywood flooring are supported by blocks to transfer loads to beams or foundation below. Beam sizes and location will be shown on engineered foundation plan. |
| RF.6 |  |  | Check for excessive gaps under header bearing points and require steel shims if necessary. |
| RF.7 |  |  | Check for number of jack studs at bearing headers per table R502.5 (1). |
| RF.8 |  |  | Check for framing anchors attached with structural fasteners at floor joists, rafters and headers. |
| RF.9 |  |  | Check notched or bored studs. Maximum 25% in exterior or bearing partitions. 40% in non-bearing partitions. 60% in exterior or bearing partitions as long as the studs are doubled and not more than 2 successive studs are bored. |
| RF.10 |  |  | Top plates joints are to be offset 24 inches. |
| RF.11 |  |  | Check for top plate laps at wall intersections and require a minimum 3x6 galvanized steel plate with six 8d nails each side if plates are not lapped. |
| RF.12 |  |  | Check for top plates cut or notched more than 50%, if so require structural straps (min 1 ½ wide) nailed with eight 16d nails each side. |
| RF.13 |  |  | Check anchor bolt spacing and anchor bolt proximity to ends of 2x4 plates. Minimum 6 feet on center and within 12 inches of plate ends. |
| RF.14 |  |  | Check for gaps between foundation plates and foundation. |
| RF.15 |  |  | Check for floating of non-bearing partitions where potential for soil expansion is moderate to high – follow recommendation of soils engineer. |
| RF.16 |  |  | Steel posts are to have no less than 1” nor more than 3” of thread showing to allow for adjustment (1”) and to assure stability (3”). (Typical engineer requirement) |
| RF.17 |  |  | Steel post top plate straps are to be bent over bottom flange of I-beam. |
| RF.18 |  |  | Check steel post base for proper bearing on pads or caissons. |
| RF.19 |  |  | Check post/beams for positive connection (beam pocket, framing anchor etc.) |
| RF.20 |  |  | Point loads bearing on TJIs over beams require that the TJI webs be filled with ¾” plywood at bearing points or squash blocks per TJI specs. |
| RF.21 |  |  | TJI blocking at bearing points that have been removed during construction must be replaced. |
| RF.22 |  |  | Check for cuts or holes in webs or chords of TJIs to see if hole chart was followed. |
| RF.23 |  |  | All cantilevers used with engineered floor systems must be solid blocked. |
| RF.24 |  |  | Structural floors in basements must be designed by a licensed professional engineer and plans are to be on the jobsite for the inspection. |
| RF.25 |  |  | Check required earth to wood clearance below wood floor joists (18”) and wood beams (12”) in crawl spaces. |
| RF.26 |  |  | Check for blocking between dimensional lumber floor joists at all bearing points including at cantilevers. |
| RF.27 |  |  | Check floor joist spans. |
| RF.28 |  |  | Check for cut floor joists and require headers where cut for tub traps, etc. |
| RF.29 |  |  | Truss clips are to be installed per engineer based on uplift on truss drawings. |
| RF.30 |  |  | Trusses are to be laterally braced at bearing points and as specified by engineer. |
| RF.31 |  |  | Check for proper bearing points where trusses cantilever such as at porches with false beams. |
| RF.32 |  |  | Check for attic access rough opening for 22” x 30” dimension and 30“ headroom measured from the plane of the ceiling to the nearest obstruction above (bottom of rafter if in 22” x 30” space or sheathing if no framing encroaches. |
| RF.33 |  |  | Check soffit vent insulation shields between trusses or rafters over exterior wall in attic and require at least 1” free air space between shield and roof sheathing. |
| RF.34 |  |  | Check for adequate roof ventilation. |
| RF.35 |  |  | Check roof trusses at roof vent holes and other cuts for top chord damage/cuts and require an engineer’s letter for damage of any sort if needed. |
| RF.36 |  |  | Check for roof deflection of long spans where framing is attached and require proper float to prevent structural damage caused by deflection. |
| RF.37 |  |  | Check roof sheathing panel rating for span. |
| RF.38 |  |  | Check roof sheathing for staggered joints. |
| RF.39 |  |  | Check rafter spans. |
| RF.40 |  |  | Check ceiling joist spans |
| RF.41 |  |  | Truss drawings must be on-site along with approved plans. |

General Requirements:

Pass|Fail

| RF.42 |  |  | Egress windows – check for egress windows in bedrooms and basement. |
| --- | --- | --- | --- |
| RF.43 |  |  | Check stair rise and run. |
| RF.44 |  |  | Check headroom at stair ceilings (6’ 8” – 6’6” at spiral stair). |
| RF.45 |  |  | Check hallway width for a minimum of 37” rough dimension, assuming ½” drywall on each side will be used (36” finished). |
| RF.46 |  |  | Check minimum ceiling height of 7 feet. |
| RF.47 |  |  | Check headroom in basement for proper clearance at beams, duct chases and main part of room. Non-habitable space minimum 6 feet 8 inches. Under beams, ducts. Minimum 6 feet 4 inches. |
| RF.48 |  |  | Check landings especially at winders for proper dimensions. Winders minimum 6 inches at any point. |
| RF.49 |  |  | Check brick veneer attachment and vapor barrier. |
| RF.50 |  |  | Brick veneer cannot support any structural loading. |
| RF.51 |  |  | Brick veneer to be supported by foundation – not on porch floor or slab. |
| RF.52 |  |  | Check for safety glazing requirements. (Glazing within 24 inches of doors and within 60 inches of the floor, glazing greater than 9 sq. ft, bottom edge within 18 inches of the floor, top edge is greater than 36 inches from floor.) |
| RF.53 |  |  | Check crawl space access minimum 18” x 24” |

Fire Blocking and Draftstop:

Fireblocking in combustible construction is required at all interconnections of hidden vertical and horizontal spaces which occur at:

Pass|Fail

| RF.54 |  |  | Kitchen soffits. |
| --- | --- | --- | --- |
| RF.55 |  |  | Drop ceilings. |
| RF.56 |  |  | Vent/duct chases at attic and ceilings. |
| RF.57 |  |  | Dead chases at attic and ceilings. |
| RF.58 |  |  | Top plate penetrations of pipes, conduit, ducts, etc. |
| RF.59 |  |  | In concealed spaces between stair stringers at the top and bottom of the run. |
| RF.60 |  |  | In fireplace chases at ceilings and attic. |
| RF.61 |  |  | Horizontally every 10 feet such as at furred spaces in basements. |
| RF.62 |  |  | Vertically such as between stud spaces and furred spaces over 10 feet in heigth. |
| RF.63 |  |  | Penetrations at floors such as tub and shower traps. |
| RF.64 |  |  | Cornices of a two-family dwelling at the line of the dwelling unit separation. |

Acceptable fireblock materials are:

Pass|Fail

| RF.65 |  |  | Fiberglass or mineral wool insulation secured in place to seal off penetrations. |
| --- | --- | --- | --- |
| RF.66 |  |  | Nominal 2x lumber fastened in place |
| RF.67 |  |  | Plywood or OSB – ¾” fastened securely in place with joints backed up by ¾” plywood or OSB. |
| RF.68 |  |  | One half inch or thicker drywall (non-rated or rated) fastened securely in place. |

Note:	The above provision is not to be confused with through-penetration firestop systems or membrane-penetration firestop systems for fire-rated assemblies.

Draftstop.

Pass|Fail

| RF.69 |  |  | Draftstop is required in combustible construction infloor-ceiling assemblies of single-family dwellings where the space exceeds 1,000 sq. ft. and where there is usable space above and below the floor. |
| --- | --- | --- | --- |

Draftstop Materials.

Pass|Fail

| RF.70 |  |  | Draftstop material shall be ½” drywall, 3/8” wood structural panel, 3/8 “particle board or other approved material properly supported. |
| --- | --- | --- | --- |
| RF.71 |  |  | Openings in draftstop within attics between units should be equipped with a means for sealing the opening. A self-closing door constructed of any structural material with a latch is acceptable – does not need fire rating. |

# ROUGH PLUMBING INSPECTION

#### DWV

Pass|Fail

| RP.1 |  |  | Check size and slope of drains. (1/4” for 2 ½” diameter and less, 1/8” for 3” and above) |
| --- | --- | --- | --- |
| RP.2 |  |  | Check for proper fittings at change of direction from vertical to horizontal, horizontal to horizontal (sweep fittings). |
| RP.3 |  |  | Check venting of traps for location, size and slope. (Every dry vent connecting to a horizontal drain shall connect above the centerline of the horizontal drain pipe. |
| RP.4 |  |  | Every dry vent shall extend 6” above flood level of fixture served before making horizontal and offset of more than 45 degrees. |
| RP.5 |  |  | Check for cleanouts at upper end of horizontal drains, at every 100 feet of run. Shall be installed at each change of direction greater than 45 degrees, except not more than one cleanout shall be required in each 40 feet of run regardless of change in direction. |
| RP.6 |  |  | Check for support of plastic pipe every 4’ O.C. horizontally and at offsets horizontally and vertically. Note: Vents in attics should be supported 4’ O.C. horizontally and where vent goes vertical even if at 45 deg. |
| RP.7 |  |  | Midstory guides required for pipe sizes 2 inches and smaller. |
| RP.8 |  |  | Check for dishwasher standpipe unless an approved airgap will be installed at fixture. |
| RP.9 |  |  | Check for clothes washer standpipe and downstream vents. |
| RP.10 |  |  | Check tub traps for proper connection. |
| RP.11 |  |  | Check for 50 lb air test on water lines or charged with water. |
| RP.12 |  |  | Check water piping for proper supports. See table P2605.1 |
| RP.13 |  |  | Check sewage ejector/lift station for proper venting. |
| RP.14 |  |  | Check vent termination above roof and proximity to openings into building. |

Gas Piping:

Pass|Fail

| RP.15 |  |  | Check gas pipe size and support. |
| --- | --- | --- | --- |
| RP.16 |  |  | Check to make sure gas pipe fittings are proper (no thread protectors allowed). |
| RP.17 |  |  | Check for exposed unions. |
| RP.18 |  |  | Check gas valve locations. (Each appliance shall have a shutoff valve located in the same room and no farther than 6’ from the appliance served) |
| RP.19 |  |  | Sediment traps shall be installed downstream of the equipment shut-off valve as close to the inlet of the equipment as practical. |
| RP.20 |  |  | Check gas pipe sizing – if not correct give option of either having pipe sized by mechanical engineer or follow sizing tables. |
| RP.21 |  |  | Observe gas pressure test of 10 PSI for 15 minutes. |
| RP.22 |  |  | If gas test passes fill out tag and attach to end of gas pipe at gas meter location. |
| RP.23 |  |  | Check remote gas shut-off for appliances. (fireplaces, bbq) |
| RP.24 |  |  | Check to see if for medium or high pressure gas will be used (check for approved regulator). |

# ROUGH  H. V. A. C.  INSPECTION

Pass|Fail

| RM.1 |  |  | Check bath fans for positive connection to the exterior. (soffit vents not allowed) (recirculating fans are not allowed) |
| --- | --- | --- | --- |
| RM.2 |  |  | Check supply ducts for proper connections. |
| RM.3 |  |  | Check return air ducts and spaces between studs and floor joists for completion. |
| RM.4 |  |  | Check that return air ducts using stud spaces are not interconnected to other floor levels. |
| RM.5 |  |  | Check for wires in plenum spaces, which run vertically along studs. |
| RM.6 |  |  | Check for return plenum spaces to be sealed from duct chases especially where flu vents are involved. |
| RM.7 |  |  | Check for return air plenum for adequate cross-sectional dimension at top plates etc. |
| RM.8 |  |  | Check between floor joists for proper and tight completion of plenums. |
| RM.9 |  |  | Check for excessive bends and length in dryer vents. Maximum 25 feet, less 5 feet for each 90 and 2 ½ feet for each 45. (no screws) |
| RM.10 |  |  | Check for proper termination of exhaust vents. |
| RM.11 |  |  | Check B vents for proper size based on equipment. |
| RM.12 |  |  | Check B vents for proper installation – no screws allowed in B vents which penetrate the inner flue pipe. |
| RM.13 |  |  | Check B-vent for clearance above roof and distance from vertical walls and openings to building. |
| RM.14 |  |  | Check for an insulation shield that extends a minimum of 2 inches above the attic insulation. |
| RM.15 |  |  | Check that vents are supported in accordance with their listing. |
| RM.16 |  |  | Check for metal fireblock pan at B vents at floors and in furnace room at ceiling and maintain 1” clearance to combustibles from B vent. |
| RM.17 |  |  | Check for combustion air duct for furnace and water heater on rough. See M1702 and M1703 or IFGC |
| RM.18 |  |  | Check B vents for clearance to combustibles at roof (roofing felt or sheathing, trusses etc.) |
| RM.19 |  |  | Check that all supply ducts and seams are made substantially tight by means of tapes, mastics or other approved closure systems. |
| RM.20 |  |  | Check listing on furnace when installed in a horizontal position (crawlspace, attic) to make sure it is an approved model for horizontal installation. |
| RM.21 |  |  | Check height above floor when appliance is installed in a garage. |
| RM.22 |  |  | Check for appliance in garage in protected from damage. |
| RM.23 |  |  | Check that rigid gas pipe is used inside appliance housing. |
| RM.24 |  |  | Check clearance and length of single wall vent connectors. |
| RM.25 |  |  | Check installation manual on high efficiency and direct vent appliances for proper vent termination. |
| RM.26 |  |  | Check attic furnaces for access catwalk and platform size. |
| RM.27 |  |  | Check that ducts in unconditioned spaces (attic) have insulation. |

# ROUGH ELECTRICAL INSPECTION

Pass|Fail

| RE.1 |  |  | Check panel driven ground connection. |
| --- | --- | --- | --- |
| RE.2 |  |  | Check laterals (burial depth, warning ribbon) |
| RE.3 |  |  | Check size and location of disconnect. |
| RE.4 |  |  | Check water pipe bonding. (must be continuous) |
| RE.5 |  |  | Check wire sizes of service entrance conductors. |
| RE.6 |  |  | Check neutral identification. |
| RE.7 |  |  | Check weather head mast height, drip loop and mast supports. |
| RE.8 |  |  | Check wire size at breaker connection, neutral is isolated from grounds at sub-panel and neutrals are not doubled under lug. |
| RE.9 |  |  | Check for minimum amount of small appliance circuits in kitchen. (2) And laundry circuit (1) |
| RE.10 |  |  | Check for nail plates where holes are closer than 11/4” from edge of plate/or framing member. |
| RE.11 |  |  | Check spacing of receptacle outlet boxes. Within 6 feet of start of wall and every 12 feet after. Must have a receptacle at any usable wall space 24 inches or greater. |
| RE.12 |  |  | Check that conductors are secured within 12 inches of all boxes. (8 “ if not supported) |
| RE.13 |  |  | Check smoke detector locations. |
| RE.14 |  |  | Check 3 wire cable for smoke detectors. |
| RE.15 |  |  | Check box fill calculation per table E3805.12.2.1 in relation to box size. |
| RE.16 |  |  | Check 220v cable to dryer or range. (4 wire) Gas dryer will only require 120v receptacle. |
| RE.17 |  |  | Check ceiling fan boxes for proper support (bridge between framing required also proper size screws). |
| RE.18 |  |  | Check switch, light wiring. White wire must be identified. |
| RE.19 |  |  | Check for 110 v W.P. (GFCI) outlet boxes - front and rear of house at grade. |
| RE.20 |  |  | Check for 110 outlet in garage. (GFCI) |
| RE.21 |  |  | Check for 110 v outlet adjacent or within 25 ft of mechanical equipment. (GFCI) |
| RE.22 |  |  | Check for light in mechanical rooms or crawl space and switch at access point. |
| RE.23 |  |  | Check that arc fault protection is supplied in all sleeping rooms. |
| RE.24 |  |  | Check for required bonding of Jacuzzi motors. |
| RE.25 |  |  | Check box or wiring in R/A stud spaces. |
| RE.26 |  |  | Check for wiring support within 6 feet of attic access. |

# DRYWALL INSPECTION

Single-Family:

Pass|Fail

| DW.1 |  |  | Check for 5/8 type X drywall inside garage when habitable space is above. (And structural members) |
| --- | --- | --- | --- |
| DW.2 |  |  | Check nail or screw spacing. See table R702.3.5 (adhesives are allowed) |
| DW.3 |  |  | Check cuts around electrical boxes to be with no more than an eight of an inch gap. |
| DW.4 |  |  | Water resistant gypsum board shall not be installed over a vapor retarder in a shower or tub area. |
| DW.5 |  |  | Enclosed accessible space under stairs shall have walls, under stair surface and any soffits protected on the enclosed side with ½ inch gypsum board. |

Multi-Family:

Pass|Fail

| DW.6 |  |  | Check fire-rating of building on plans and review fire test data before inspection. |
| --- | --- | --- | --- |
| DW.7 |  |  | If building is non-rated there may still be area separation walls and in any event 1-hour unit separation will be required at partition walls and floor/ceilings if condos or apartments. |
| DW.8 |  |  | Often a “pre-rock” rough framing, plumbing HVAC and electrical inspection may need to be done for areas such as drop ceiling areas, behind tubs and showers, behind duct chases etc. |
| DW.9 |  |  | If an area separation wall is part of the inspection, the base layer and finish layer will have to be inspected individually to assure that proper staggering and fastening schedules are followed. This is true for both walls and for multi-layer floor or roof/ceiling assemblies. |
| DW.10 |  |  | If a common 2-Hour wall is constructed between units, no plumbing, mechanical, ducts or vents are allowed in wall. |
| DW.11 |  |  | Check nailing patterns to conform to fire tested assembly detail. |
| DW.12 |  |  | Check for gaps around penetrations such as wood framing, metal pipes, ducts and steel outlet boxes and do not allow more than ¼” gaps. For plastic electrical outlet boxes make sure the boxes are listed for the assembly it is a part of and follow the listing – typically the maximum gap allowed is 1/8”. |
| DW.13 |  |  | Check for plastic pipe and conduit penetrations. These penetrations are not allowed without either a through-penetration firestop or membrane-penetration listed firestop system. Ask for specification of firestop system to be used. |
| DW.14 |  |  | Check for continuation of fire-rating through attics, floor/ceiling assemblies, etc. at exterior walls. |
| DW.15 |  |  | Check fire-rated shaft wall termination to be to the underside of roof sheathing unless top of shaft is enclosed with same fire-rating as walls. |
| DW.16 |  |  | Check corridor walls and ceiling for complete envelope. Corridor walls must extend to floor or roof deck or be tented. |

##### FINAL BUILDING INSPECTION

Pass|Fail

| FB.1 |  |  | Check for address numbers visible from the street on contrasting background. |
| --- | --- | --- | --- |
| FB.2 |  |  | Check for flashing and counter flashing at exterior. |
| FB.3 |  |  | Check exterior penetrations of siding such as gas pipe TV cable etc. and require seal. |
| FB.4 |  |  | Check for positive drainage away from building around house (2% or ¼ inch per foot). |
| FB.5 |  |  | Check step rise and run and check for gripable handrails where required. |
| FB.6 |  |  | Check for door swing over landings or steps not allowed. (screen or storm doors are allowed) |
| FB.6 |  |  | Check egress window size (5.7 Sq. Ft. openable area, at least 20” wide, 24” high) and height (not over 44” above floor to sill). |
| FB.8 |  |  | Check window wells at egress windows for exit criteria (36 inch clear space outside of basement windows. Ladder required where window well is deeper than 44 inches). |
| FB.9 |  |  | Check Smoke detector locations and test to see if functioning and interconnected. |
| FB.10 |  |  | Check crawl space for proper ventilation. |
| FB.11 |  |  | Check under-stair areas for ½ inch drywall if considered usable space. |
| FB.12 |  |  | Check insulation of rim joists. |
| FB.13 |  |  | Check beam bearing in basement. Minimum 3 inches for beams in concrete |
| FB.14 |  |  | Check to see if steel posts are secure to I beam. |
| FB.15 |  |  | Check for cut floor joists in basement at traps etc. |
| FB.16 |  |  | Check exterior landings and stairs at decks and porches. |
| FB.17 |  |  | Check guards at areas greater than 30 inches above floor or grade. (minimum 36 inches) |
| FB.18 |  |  | Check for approved attic access. Not located over closet shelves. |
| FB.19 |  |  | Check attic space for proper insulation depth. |
| FB.20 |  |  | Check for appliances located in the attic. If so check for 24 inch wide unobstructed walkway to 30 inch clear working platform, service receptacle and light, switched from access opening. |
| FB.21 |  |  | Check showers and shower/tubs are finished for 72 inches above the floor with nonabsorbent material. |
| FB.22 |  |  | Check for required safety glazing. |

# FINAL PLUMBING INSPECTION

Pass|Fail

| FP.1 |  |  | Check to see that all fixtures are connected to plumbing properly. |
| --- | --- | --- | --- |
| FP.2 |  |  | Check plumbing fixtures – are hot and cold crossed? |
| FP.3 |  |  | Check that all test caps are removed on roof vents. |
| FP.4 |  |  | Test stools. |
| FP.5 |  |  | Check for minimum clear space at fixtures. |
| FP.6 |  |  | Test showers |
| FP.7 |  |  | Test garbage disposal. |
| FP.8 |  |  | Check to see if dishwasher discharge hose is connected to a standpipe or an approved air gap. (and secured) |
| FP.9 |  |  | Check exterior gas piping – to be painted to prevent corrosion. |
| FP.10 |  |  | Check hose bibs – are hose bibs equipped with back-flow prevention? |
| FP.11 |  |  | Check jetted tub for access to pump. |
| FP.12 |  |  | Check seal of shower fixture to tile or surround. |
| FP.13 |  |  | Check for open waste lines in basement. |
| FP.14 |  |  | Check for dielectric unions at water heater. |
| FP.15 |  |  | Check for pressure relief valve on water heater and where it terminates. |
| FP.16 |  |  | Check that fixtures located in basement are protected with an approved backflow device. |

FINAL H.V.A.C. INSPECTION

Pass|Fail

| FM.1 |  |  | Check gas fireplace for proper function and for accessible gas pipe valve under unit. |
| --- | --- | --- | --- |
| FM.2 |  |  | Check furnace function (venting gas connection etc.). |
| FM.3 |  |  | Check water heater function venting, gas connection, etc.). |
| FM.4 |  |  | Check bath fan function. |
| FM.5 |  |  | Check kitchen hood function. |
| FM.6 |  |  | Check combustion air ducts for proper location size and termination. |
| FM.7 |  |  | Check flue vent termination and caps. |
| FM.8 |  |  | Check fire block at flue penetration of ceiling or floor. |
| FM.9 |  |  | Check return air duct to furnace connection for gaps. |
| FM.10 |  |  | Check return air plenum caps between joists for excessive gaps |

# FINAL ELECTRICAL INSPECTION

Pass|Fail

| FE.1 |  |  | Test required GFCI’s in baths, kitchen counters (2 circuits), garage’s, exterior outlets, basement and in attic if needed. |
| --- | --- | --- | --- |
| FE.2 |  |  | Check panel for circuit labels, grounding, etc. |
| FE.3 |  |  | Check arch fault breaker in panel. |
| FE.4 |  |  | Check water pipe ground connection. |
| FE.5 |  |  | Check fixtures for proper connection and function. |
| FE.6 |  |  | Check final spacing of receptacles in habitable rooms and spaces. |
| FE.7 |  |  | Check A/C connection, wiring and max breaker size. |
| FE.8 |  |  | Check disposal/dishwasher 2 pole breaker if using 1 duplex device. |
| FE.9 |  |  | Check meter/disconnect/panel closures. |
| FE.10 |  |  | Check that ground rod is flush with grade |
| FE.11 |  |  | Check smoke detectors with power and no power and AF protected. |
| FE.12 |  |  | Check light outside garage service door operable |
| FE.13 |  |  | Check spark rings for microwave and disposal-most common receptacles set back, not flush with finished surface. |